Information regarding your files

Microprobe:

Basalt glass measurements: 3 files (1 file with the oxide data and 2 files from which you can read your measurement conditions/standards).

Olivine measurements: 5 files (1 file with the oxide data, 1 file with element data, 1 file with Forsterite/Fayalite ratios, 2 files for the measurement conditions)

Regarding the numbering of the samples: the last number in a sample name indicates the measurement: so if all numbers in the name are the same apart from the last number it is multiple measurements of 1 sample.

For the glass data there were 9 measurements per sample. The Olivine data has an irregular amount of points per mineral grain and sometimes they are measured with a transect (those points start with ‘Line’).

In microprobe the standards are: SC-Ol for Ol (3x) and VG2, VGA99 and Scap for glasses (check the measurements conditions files to see which elements are calibrated on which standard). Here the samples are already calibrated in the machine, but you need to check if the data is correct. In the calibration most elements in Ol are calibrated on SC-Ol, in the case of the glass, most elements are calibrated on VG2, so at least this needs to be correct. VG2 and VGA99 are two basalts, Scap is another mineral - so many elements will be off (as it has a different matrix), but it is included because some elements are calibrated on Scapolite (check in the method document) as they are too low in the basalts.

LAICPMS:

Most of the measurements are individual ‘samples’ as it is melt inclusions. If there are duplicates (multiple measurements of the same sample) its indicated with A,B,C (sometimes A is missing)

- The standards used to calibrate the measurements on is KL2

- The other standards (all that do not start with MC) are added as ‘unknowns’ to be able to check the precision and accuracy of the measurements.

- Standard NIST, was just measured to check the overall performance of the machine, you can ignore this one (or also use to check for accuracy)

- In general I measured according to the following schedule:

NIST standards

3 KL2 standards to calibrate the samples

20 samples (including several other standards measured as sample, e.g. GOR, BM.., BCR and also KL2): these are not to calibrate the samples but to check for accuracy

3 KL2 standards

20 samples

…

Etc..

…

3 KL2 standards

NIST standards